What You Need to Know:

Skill Sheet: Areas

Examiners

Top Tip

The area of a complicated shape can be calculated by breaking it down into a number of simple shapes

The area of a component may be needed to calculate, for example:

- the amount of material required to make a number of parts;

- how much waste will be created when a part is cut out;

- the volume or mass of material needed to make a product.

The formulae for the areas of simple shapes are:

Area of a rectangle = length x width = L x W

Area of a triangle= half (base x height) = ½ (B x H)

Area of a circle = π r2

A square is a rectangle where the length and width are the same. A cube is a 3D square with six faces. Therefore the surface area of a cube = 6 x length x length = 6 x L2.

Example:

Calculate the area of the part shown in figure 1.

Answer:

Treat the shape as 2 parts: a rectangle and a semi-circle

Area of the rectangle = L x W = 200 x 100 = 2 x 104 mm2

Figure 1

Area of the semi-circle = ½ π r2 = ½ x 3.14 x 1002 = 1.57 x 104 mm2

Total area = 2 x 104 + 1.57 x 104 = 3.57 x 104 mm2

Now Try These:

Figure 2

30 mm

110 mm

- Calculate the area of the part in Figure 2.

___________________________________________________________________________________________

___________________________________________________________________________________________

- The shape in Figure 3 needs to be cut out for use in a repair. Calculate the area of the shape.

NOT TO SCALE

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

Figure 3

Now Try These:

Practice Sheet: Areas

- A rectangle with sides of 450 mm and 200 mm needs to be cut to make a side panel for a machine.

Calculate the area of a rectangle, using standard form.

___________________________________________________________________________________________

___________________________________________________________________________________________

- Calculate the surface area of a cube where each side is 0.6 m.

___________________________________________________________________________________________

___________________________________________________________________________________________

- Calculate the area of the shape shown in Figure 1.

NOT TO SCALE

____________________________________________________________________

____________________________________________________________________

____________________________________________________________________

____________________________________________________________________

Figure 1

____________________________________________________________________

- A manufacturer has a rectangular sheet of steel that is 1.2 m by 1.4 m. This will be used to cut out four circles of radius 0.3 m. Once the circles have been cut out the remaining material will be scrap.

- Determine the area of one circle.

________________________________________________________________________________________

________________________________________________________________________________________

- Calculate the percentage of material that will be scrap.

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

________________________________________________________________________________________

Answers:

Skill Sheet: Areas

- Area = ½ (B x H) = ½ (30 x 110) = 1650 mm2

- The shape is made up of a rectangle and a triangle

Area of the rectangle = L x W = 1.2 x 0.9 = 1.08 m2

Area of the triangle = ½ (B x W) = ½ (0.9 x 0.4) = 0.18 m2

Total area = 1.08 + 0.18 = 1.26 m2

Practice Sheet: Areas

- Area of the rectangle = L x W = 450 x 200 = 90000 mm2= 9 x 104 mm2

- Area of the cube = 6 x L2 = 6 x 0.36 = 2.16 m2

- The shape is made up of a semi-circle and a triangle

Area of the semi-circle = π r2 / 2 = 3.14 x 0.42 /2 = 0.251 m2

Area of the triangle = ½ (B x W) = ½ (0.8 x 0.6) = 0.24 m2

Total area = 0.251 + 0.24 = 0.491 m2

- a) Area = π r2 = 3.14 x 0.32 = 0.2826 m2

b) Area of the sheet = L x W = 1.2 x 1.4 m2 = 1.68 m2

Total area of the 4 circles = 4 x 0.2826 = 1.1304 m2

Waste = 1.68 – 1.1304 = 0.5496 m2

% waste = 0.5496 / 1.68 x 100 / 1 = 32.71 %